SUBELEMENT T3  –  Operating practices – 4 exam questions – 4 groups

T3A - Choosing an operating frequency, calling CQ, calling another station, test transmissions – 1 exam question

T3A01 (B)

Which of the following should you do when selecting a frequency on which to transmit?

A. Call CQ to see if anyone is listening

B. Listen to determine if the frequency is busy

C. Transmit on a frequency that allows your signals to be heard

D. Check for maximum power output

~~

T3A02 (B)

How do you call another station on a repeater if you know the station's call sign?

A. Say "break, break" then say the station's call sign

B. Say the station's call sign then identify your own station

C. Say "CQ" three times then the other station's call sign

D. Wait for the station to call "CQ" then answer it

~~

T3A03 (A)

How do you indicate you are looking for any station with which to make contact?

A. CQ followed by your callsign

B. RST followed by your callsign

C. QST followed by your callsign

D. SK followed by your callsign

~~

T3A04 (C)

What should you transmit when responding to a call of CQ?

A. Your own CQ followed by the other station’s callsign

B. Your callsign followed by the other station’s callsign

C. The other station’s callsign followed by your callsign

D. A signal report followed by your callsign

~~

T3A08 (D)

What is the meaning of the procedural signal "CQ"?

A. Call on the quarter hour

B. New antenna is being tested (no station should answer)

C. Only the called station should transmit

D. Calling any station

~~

T3A09 (A) [97.119(b)(2)]

Why should you avoid using cute phrases or word combinations to identify your station?

A. They are not easily understood by some operators

B. They might offend some operators

C. They do not meet FCC identification requirements

D. They might be interpreted as codes or ciphers intended to obscure your identification

~~

T3A10 (B)

What brief statement is often used in place of "CQ" to indicate that you are listening for calls on a repeater?

A. Say "Hello test" followed by your call sign

B. Say your call sign

C. Say the repeater call sign followed by your call sign

D. Say the letters "QSY" followed by your call sign

~~

T3A11 (A) [97.119(b)(2)]

Why should you use the International Telecommunication Union (ITU) phonetic alphabet when identifying your station?

A. The words are internationally recognized substitutes for

letters

B. There is no advantage

C. The words have been chosen to represent amateur radio terms

D. It preserves traditions begun in the early days of amateur radio

~~

T3B - Use of minimum power, band plans, repeater coordination, mode restricted sub-bands – 1 exam question

T3B01 (A)

What is a band plan?

A. A voluntary guideline, beyond the divisions established by the FCC for using different operating modes within an amateur band

B. A guideline from the FCC for making amateur frequency band allocations

C. A guideline for operating schedules within an amateur band published by the FCC

D. A plan devised by a local group

~~

T3B02 (C)

Which of the following statements is true of band plans?

A. They are mandated by the FCC to regulate spectrum use

B. They are mandated by the ITU

C. They are voluntary guidelines for efficient use of the radio spectrum

D. They are mandatory only in the US

~~

T3B03 (C)

Who developed the band plans used by amateur radio operators?

A. The US Congress

B. The FCC

C. The amateur community

D. The Interstate Commerce Commission

~~

T3B04 (C)

Who is in charge of the repeater frequency band plan in your local area?

A. The local FCC field office

B. RACES and FEMA

C. The recognized frequency coordination body

D. Repeater Council of America

~~

T3B05 (A)

What is the main purpose of repeater coordination?

A. To reduce interference and promote proper use of spectrum

B. To coordinate as many repeaters as possible in a small area

C. To coordinate all possible frequencies available for repeater use

D. To promote and encourage use of simplex frequencies

~~

T3B06 (C) [97.205(g)]

Who is accountable if a repeater station inadvertently retransmits communications that violate FCC rules?

A. The repeater trustee

B. The repeater control operator

C. The transmitting station

D. All of these answers are correct

~~

T3B07 (D) [97.313(a)]

Which of these statements is true about legal power levels on the amateur bands?

A. Always use the maximum power allowed to ensure that you

complete the contact

B. An amateur may use no more than 200 Watts PEP to make an

amateur contact

C. An amateur may use up to 1500 Watts PEP on any amateur

frequency

D. An amateur must use the minimum transmitter power necessary to carry out the desired communication

~~

T3C - Courtesy and respect for others, sensitive subject areas, obscene and indecent language – 1 exam question

T3C01 (A)

What is the proper way to break into a conversation between two stations that are using the frequency?

A. Say your call sign between their transmissions

B. Wait for them to finish and then call CQ

C. Say "Break-break" between their transmissions

D. Call one of the operators on the telephone to interrupt the conversation

~~

T3C02 (D)

What is considered to be proper repeater operating practice?

A. Monitor before transmitting and keep transmissions short

B. Identify legally

C. Use the minimum amount of transmitter power necessary

D. All of these answers are correct

~~

T3C03 (A)

What should you do before responding to another stations call?

A. Make sure you are operating on a permissible frequency for your license class

B. Adjust your transmitter for maximum power output

C. Ask the station to send their signal report and location

D. Verify the other station’s license class

~~

T3C04 (C) [97.101(b)]

What rule applies if two amateur stations want to use the same frequency?

A. The station operator with a lesser class of license must yield the frequency to a higher-class licensee

B. The station operator with a lower power output must yield the frequency to the station with a higher power output

C. No frequency will be assigned for the exclusive use of any station and neither has priority

D. Station operators in ITU Regions 1 and 3 must yield the

frequency to stations in ITU Region 2

~~

T3C05 (D) [97.113(a)(4)]

Why is indecent and obscene language prohibited in the Amateur Service?

A. Because it is offensive to some individuals

B. Because young children may intercept amateur communications with readily available receiving equipment

C. Because such language is specifically prohibited by FCC Rules

D. All of these choices are correct

~~

T3C06 (B)

Why should amateur radio operators avoid the use of racial or ethnic slurs when talking to other stations?

A. Such language is prohibited by the FCC

B. It is offensive to some people and reflects a poor public image on all amateur radio operators

C. Some of the terms used may be unfamiliar to other operators

D. You transmissions might be recorded for use in court

~~

T3C07 (C)

What should you do if you hear a newly licensed operator that is having trouble with their station?

A. Tell them to get off the air until they learn how operate properly

B. Report them to the FCC

C. Contact them and offer to help with the problem

D. Move to another frequency

~~

T3C08 (B)

Where can an official list be found of prohibited obscene and indecent words that should not be used in amateur radio? 

A. On the FCC web site 
B. There is no official list of prohibited obscene and indecent words 
C. On the Department of Commerce web site 
D. The official list is in public domain and found in all amateur study guides

~~

T3C09 (D) [97.113(a)(4)]
What type of subjects are not prohibited communications while using amateur radio? 

A. Political discussions
B. Jokes and stories 
C. Religious preferences
D. All of these answers are correct

~~

T3C10 (C) [97.101 (a)]
When circumstances are not specifically covered by FCC rules, what general operating standard must be applied to amateur station operation? 

A. Designated operator control 
B. Politically correct control 
C. Good engineering and amateur practices 
D. Reasonable operator control

~~

T3D - Interference to and from consumer devices, public relations, intentional and unintentional interference - 1 exam question

T3D06 (D)

Who has exclusive use of a specific frequency when the FCC has not declared a communication emergency?

A. Any net station that has traffic

B. The station first occupying the frequency

C. Individuals passing health and welfare communications

D. No station has exclusive use of any frequency

~~

T3D09 (C) [97.103(a)]

What rules apply to your station when using amateur radio at the request of public service officials or at the scene of an emergency?

A. RACES

B. ARES

C. FCC

D. FEMA

~~

T3D10 (D)

What do RACES and ARES have in common?

A. They represent the two largest ham clubs in the United States

B. One handles road traffic, the other weather traffic

C. Neither may handle emergency traffic

D. Both organizations provide communications during emergencies

~~

SUBELEMENT T5  –  Station setup and operation - 4 exam questions – 4 groups

T5C – Repeaters; repeater and simplex operating techniques, offsets, selective squelch, open and closed repeaters, linked repeaters - 1 exam question

T5C02 (B)

What is a courtesy tone?

A. A tone used to identify the repeater

B. A tone used to indicate when a transmission is complete

C. A tone used to indicate that a message is waiting for someone

D. A tone used to activate a receiver in case of severe weather

~~

T5C04 (C)

Why should you pause briefly between transmissions when using a repeater?

A. To let your radio cool off

B. To reach for pencil and paper so you can take notes

C. To listen for anyone wanting to break in

D. To dial up the repeater's autopatch

~~

T5C09 (B)

What is a reason to use simplex instead of a repeater?

A. When the most reliable communications are needed

B. To avoid tying up the repeater when direct contact is possible

C. When an emergency telephone call is needed

D. When you are traveling and need some local information

~~

T5C10 (A)

How might you find out if you could communicate with a station using simplex instead of a repeater?

A. Check the repeater input frequency to see if you can hear the other station

B. Check to see if you can hear the other station on a different frequency band

C. Check to see if you can hear a more distant repeater

D. Check to see if a third station can hear both of you

~~

T5C11 (C)

What is the term for a series of repeaters that can be connected to one another to provide users with a wider coverage?

A. Open repeater system

B. Closed repeater system

C. Linked repeater system

D. Locked repeater system

~~

T5C12 (A)

What is the main reason repeaters should be approved by the local frequency coordinator before being installed?

A. Coordination minimizes interference between repeaters and makes the most efficient use of available frequencies

B. Coordination is required by the FCC

C. Repeater manufacturers have exclusive territories and you could be fined for using the wrong equipment

D. Only coordinated systems will be approved by the officers of the local radio club

~~

T5C13 (B)

Which of the following statements regarding use of repeaters is true?

A. All amateur radio operators have the right to use any repeater at any time

B. Access to any repeater may be limited by the repeater owner

C. Closed repeaters must be opened at the request of any amateur wishing to use it

D. Open repeaters are required to use CTCSS tones for access

~~

T5C14 (D)

What term is used to describe a repeater when use is restricted to the members of a club or group?

A. A beacon station

B. An open repeater

C. A auxiliary station

D. A closed repeater

~~

T5D – Recognition and correction of problems, symptoms of overload and overdrive, distortion, over and under modulation, RF feedback, off frequency signals, fading and noise, problems with digital

communications links – 1 exam question

T5D11 (C)

What may be the problem if another operator reports that your SSB signal is very garbled and breaks up?

A. You have the noise limiter turned on

B. The transmitter is too hot and needs to cool off

C. RF energy may be getting into the microphone circuit and

causing feedback

D. You are operating on lower sideband

~~

T5D12 (D)

What might be the problem if you receive a report that your signal through the repeater is distorted or weak?

A. Your transmitter may be slightly off frequency

B. Your batteries may be running low

C. You could be in a bad location

D. All of these answers are correct

~~

T5D13 (B)

What is one of the reasons to use digital signals instead of analog signals to communicate with another station?

A. Digital systems are less expensive than analog systems

B. Many digital systems can automatically correct errors caused by noise and interference

C. Digital modulation circuits are much less complicated than any other types

D. All digital signals allow higher transmit power levels

~~

SUBELEMENT T6 – Communications modes and methods – 3 exam questions - 3 groups

T6A - Modulation modes, descriptions and bandwidth (AM, FM, SSB) – 1 exam question

T6A11 (B)

What is the normal bandwidth required for a conventional fast-scan TV transmission using combined video and audio on the 70-centimeter band?

A. More than 10 MHz

B. About 6 MHz

C. About 3 MHz

D. About 1 MHz

~~

T6B - Voice communications,  EchoLink and IRLP – 1 exam question

T6B01 (C)

How is information transmitted between stations using Echolink?

A. APRS

B. PSK31

C. Internet

D. Atmospheric ducting

~~

T6B02 (A)

What does the abbreviation IRLP mean?

A. Internet Radio Linking Project

B. Internet Relay Language Protocol

C. International Repeater Linking Project

D. International Radio Linking Project

~~

T6B03 (B)

Who may operate on the Echolink system?

A. Only club stations

B. Any licensed amateur radio operator

C. Technician class licensed amateur radio operators only

D. Any person, licensed or not, who is registered with the

Echolink system

~~

T6B04 (A)

What technology do Echolink and IRLP have in common?

A. Voice over Internet protocol

B. Ionospheric propagation

C. AC power lines

D. PSK31

~~

T6B05 (C)

What method is used to transfer data by IRLP?

A. VHF Packet radio

B. PSK31

C. Voice over Internet protocol

D. None of these answers are correct

~~

T6B06 (B)

What does the term IRLP describe?

A. A method of encrypting data

B. A method of linking between two or more amateur stations using the Internet

C. A low powered radio using infra-red frequencies

D. An international logging program.

~~

T6B07 (B)

Which one of the following allows computer-to-radio linking for voice transmission?

A. Grid modulation

B. EchoLink

C. AMTOR

D. Multiplex

~~

T6B08 (C)

What are you listening to if you hear a brief tone and then a station from Russia calling CQ on a 2-meter repeater?

A. An ionospheric band opening on VHF

B. A prohibited transmission

C. An Internet linked DX station

D. None of these answers are correct

~~

T6B10 (C)

Where might you find a list of active nodes using VoIP?

A. The FCC Rulebook

B. From your local emergency coordinator

C. A repeater directory or the Internet

D. The local repeater frequency coordinator

~~

T6B11 (D)

When using a portable transceiver how do you select a specific IRLP node?

A. Choose a specific CTCSS tone

B. Choose the correct DSC tone

C. Access the repeater autopatch

D. Use the keypad to transmit the IRLP node numbers

~~

T6C – Non-voice communications - image communications, data, CW, packet, PSK31, Morse code techniques, Q signals – 1 exam question

T6C01 (D)

Which of the following is an example of a digital communications method?

A. Single sideband voice

B. Amateur television

C. FM voice

D. Packet radio

~~

T6C02 (A)

What does the term APRS mean?

A. Automatic Position Reporting System

B. Associated Public Radio Station

C. Auto Planning Radio Set-up

D. Advanced Polar Radio System

~~

T6C03 (D)

What item is required along with your normal radio for sending automatic location reports?

A. A connection to the vehicle speedometer

B. A connection to a WWV receiver

C. A connection to a broadcast FM sub-carrier receiver

D. A global positioning system receiver

T6C04 (C)

What type of transmission is indicated by the term NTSC?

A. A Normal Transmission mode in Static Circuit

B. A special mode for earth satellite uplink

C. A standard fast scan color television signal

D. A frame compression scheme for TV signal

~~

T6C06 (B)

What does the abbreviation PSK mean?

A. Pulse Shift Keying

B. Phase Shift Keying

C. Packet Short Keying

D. Phased Slide Keying

~~

T6C07 (D)

What is PSK31?

A. A high-rate data transmission mode used to transmit files

B. A method of reducing noise interference to FM signals

C. A type of television signal

D. A low-rate data transmission mode that works well in noisy conditions

~~

T6C08 (C)

What sending speed is recommended when using Morse code?

A. Only speeds below five WPM

B. The highest speed your keyer will operate

C. Any speed at which you can reliably receive

D. The highest speed at which you can control the keyer

~~

T6C09 (D)

What is a practical reason for being able to copy CW when using repeaters?

A. To send and receive messages others cannot overhear

B. To conform with FCC licensing requirements

C. To decode packet radio transmissions

D. To recognize a repeater ID sent in Morse code

~~

T6C10 (A)

What is the "Q" signal used to indicate that you are receiving interference from other stations?

A. QRM

B. QRN

C. QTH

D. QSB

~~

T6C11 (B)

What is the "Q" signal used to indicate that you are changing frequency?

A. QRU

B. QSY

C. QSL

D. QRZ

~~

SUBELEMENT T7 – Special operations – 2 exam questions – 2 groups

T7A – Operating in the field, radio direction finding, radio control, contests, special event stations – 1 exam question

T7A01 (C)

What is a good thing to have when operating a hand-held transceiver away from home?

A. A selection of spare parts

B. A programming cable to load new channels

C. One or more fully charged spare battery packs

D. A dummy load

~~

T7A02 (B)

Which of these items would probably not be very useful to include in an emergency response kit?

A. An external antenna and several feet of connecting cable

B. A 1500 watt output linear amplifier

C. A cable and clips for connecting your transceiver to an

external battery

D. A listing of repeater frequencies and nets in your area

~~

T7A04 (C)

What would be a good thing to have when operating from a location that includes lots of crowd noise?

A. A portable bullhorn

B. An encrypted radio

C. A combination headset and microphone

D. A pulse noise blanker

~~

T7A05 (C)

What is a method used to locate sources of noise interference or jamming?

A. Echolocation

B. Doppler radar

C. Radio direction finding

D. Phase locking

~~

T7A06 (B)

Which of these items would be the most useful for a hidden transmitter hunt?

A. Binoculars and a compass

B. A directional antenna

C. A calibrated noise bridge

D. Calibrated SWR meter

~~

T7A07 (A)

What is a popular operating activity that involves contacting as many stations as possible during a specified period of time?

A. Contesting

B. Net operations

C. Public service events

D. Simulated emergency exercises

~~

T7A09 (A)

What is a grid locator?

A. A letter-number designator assigned to a geographic location

B. Your azimuth and elevation

C. Your UTC location

D. The 4 digits that follow your ZIP code

~~

T7A10 (C)

What is a special event station?

A. A station that sends out birthday greetings

B. A station that operates only on holidays

C. A temporary station that operates in conjunction with an

activity of special significance

D. A station that broadcasts special events

~~

T7A11 (B) [97.215(c)]

What is the maximum power allowed when transmitting telecommand signals to radio controlled models?

A. 500 milliwatts

B. 1 watt

C. 25 watts

D. 1500 watts

~~

T7A12 (C) [97.215(a)]

What is the station identification requirement when sending commands to a radio control model using amateur frequencies?

A. Voice identification must be transmitted every 10 minutes

B. Morse code ID must be sent once per hour

C. A label indicating the licensee's call sign and address must be affixed to the transmitter

D. There is no station identification requirement for this service

~~

T7B – Satellite operation, Doppler shift, satellite sub bands, LEO,  orbit calculation, split frequency operation, operating protocols, AMSAT, ISS communications – 1 exam question

T7B01 (D)

What class of license is required to use amateur satellites?

A. Only Extra class licensees can use amateur radio satellites

B. General or higher class licensees who have a satellite operator certification

C. Only persons who are AMSAT members and who have paid their dues

D. Any amateur whose license allows them to transmit on the

satellite uplink frequency

~~

T7B02 (B)

How much power should you use to transmit when using an amateur satellite?

A. The maximum power of your transmitter

B. The minimum amount of power needed to complete the contact

C. No more than half the rating of your linear amplifier

D. Never more than 1 watt

~~

T7B03 (D)

What is something you can do when using an amateur radio satellite?

A. Listen to the Space Shuttle

B. Get global positioning information

C. Make autopatch calls

D. Talk to amateur radio operators in other countries

~~

T7B04 (B)

Who may make contact with an astronaut on the International Space Station using amateur radio frequencies?

A. Only members of amateur radio clubs at NASA facilities

B. Any amateur with a Technician or higher class license

C. Only the astronaut's family members who are hams

D. You cannot talk to the ISS on amateur radio frequencies

~~

T7B05 (D)

What is a satellite beacon?

A. The primary transmit antenna on the satellite

B. An indicator light that that shows where to point your antenna

C. A reflective surface on the satellite

D. A signal that contains information about a satellite

~~

T7B06 (D)

What should you use to determine when you can access an amateur satellite?

A. A GPS receiver

B. A field strength meter

C. A telescope

D. A satellite tracking program

~~

T7B07 (C)

What is Doppler shift?

A. A change in the satellite orbit

B. A mode where the satellite receives signals on one band and transmits on another

C. A change in signal frequency caused by motion through space

D. A special digital communications mode for some satellites

~~

T7B08 (C)

What is the name of the group that coordinates the building and/or  launch of the largest number of amateur radio satellites?

A. NSA

B. USOC

C. AMSAT

D. FCC

~~

T7B09 (C)

What is a satellite sub-band?

A. A special frequency for talking to submarines

B. A frequency range limited to Extra Class licensees

C. A portion of a band where satellite operations are permitted

D. An obsolete term that has no meaning

~~

T7B10 (B)

What is the satellite sub-band on 70-CM?

A. 420 to 450 MHz

B. 435 to 438 MHz

C. 440 to 450 MHz

D. 432 to 433 MHz

~~

T7B11 (C)

What do the initials LEO tell you about an amateur satellite?

A. The satellite battery is in Low Energy Operation mode

B. The satellite is performing a Lunar Ejection Orbit maneuver

C. The satellite is in a Low Earth Orbit

D. The satellite uses Light Emitting Optics

~~

SUBELEMENT T8 – Emergency and Public Service Communications – 3 exam questions – 3 groups

T8A - FCC declarations of an emergency, use of non-amateur equipment and frequencies, use of equipment by unlicensed persons, tactical call signs – 1 exam question

T8A01 (C) [97.401(b)]

What information is included in an FCC declaration of a temporary state of communication emergency?

A. A list of organizations authorized to use radio communications in the affected area

B. A list of amateur frequency bands to be used in the affected area

C. Any special conditions and rules to be observed during the emergency

D. An operating schedule for authorized amateur emergency stations

~~

T8A02 (B) [97.111(a)]

Under what conditions are amateur stations allowed to communicate with stations operating in other radio services?

A. When communicating with the space shuttle

B. When specially authorized by the FCC, or in an actual emergency

C. When communicating with stations in the Citizens Radio Service

D. When a commercial broadcast station is reporting news during a natural disaster

~~

T8A03 (D)

What should you do if you are in contact with another station and an emergency call is heard?

A. Tell the calling station that the frequency is in use

B. Direct the calling station to the nearest emergency net

frequency

C. Disregard the call and continue with your contact

D. Stop your contact immediately and take the emergency call

~~

T8A04 (C)

What are the restrictions on amateur radio communications after the FCC has declared a communications emergency?

A. The emergency declaration prohibits all communications

B. There are no restrictions if you have a special emergency certification

C. You must avoid those frequencies dedicated to supporting

the emergency unless you are participating in the relief effort

D. Only military stations are allowed to use the amateur radio frequencies during an emergency

~~

T8A05 (B)

What is one reason for using tactical call signs such as "command post" or "weather center" during an emergency?

A. They help to keep the general public informed

B. They are more efficient and help coordinate public-service communications

C. They are required by the FCC

D. They increase goodwill and sound professional

T8A06 (A) [97.401(b)]

What is legally required to restrict a frequency to emergency-only communication?

A. An FCC declaration of a communications emergency

B. Determination by the designated net manager for an emergency net

C. Authorization by an ARES/RACES emergency coordinator

D. A Congressional declaration of intent

~~

T8A07 (D)

Who has the exclusive use of a frequency if the FCC has not declared a communication emergency?

A. Any net station that has traffic

B. The station first occupying the frequency

C. Individuals passing health and welfare communications

D. No station has exclusive use in this circumstance

~~

T8A08 (B)

What should you do if you hear someone reporting an emergency?

A. Report the station to the FCC immediately

B. Assume the emergency is real and act accordingly

C. Ask the other station to move to a different frequency

D. Tell the station to call the police on the telephone

~~

T8A09 (D)

What is an appropriate way to initiate an emergency call on amateur radio?

A. Yell as loudly as you can into the microphone

B. Ask if the frequency is in use and wait for someone to

give you permission to go ahead before proceeding

C. Declare a communications emergency

D. Say "Mayday, Mayday, Mayday" followed by "any station come in please" and identify your station

~~

T8A10 (D)

What are the penalties for making a false emergency call?

A. You could have your license revoked

B. You could be fined a large sum of money

C. You could be sent to prison

D. All of these answers are correct

T8A11 (B) [97.101(c)]

What type of communications has priority at all times in the Amateur Radio Service?

A. Repeater communications

B. Emergency communications

C. Simplex communications

D. Third-party communications

~~

T8A12  (D)  [97.101(c)]
When must priority be given to stations providing emergency communications? 

A.  Only when operating under RACES 
B.  Only when an emergency has been declared 
C.  Any time a net control station is on the air 
D.  At all times and on all frequencies

~~

T8B - Preparation for emergency operations, RACES/ARES, safety of life and property, using ham radio at civic events, compensation prohibited – 1 exam question

T8B01 (D)

What can you do to be prepared for an emergency situation where your assistance might be needed?

A. Check at least twice a year to make sure you have all of your emergency response equipment and know where it is

B. Make sure you have a way to run your equipment if there is a power failure in your area

C. Participate in drills that test your ability to set up and operate in the field

D. All of these answers are correct

~~

T8B02 (C) [97.403]

When may you use your amateur station to transmit a "SOS" or "MAYDAY" signal?

A. Only when you are transmitting from a ship at sea

B. Only at 15 and 30 minutes after the hour

C. When there is immediate threat to human life or property

D. When the National Weather Service has announced a weather warning

~~

T8B03 (A)

What is the primary function of RACES in relation to emergency activities?

A. RACES organizations are restricted to serving local, state, and federal government emergency management agencies

B. RACES supports agencies like the Red Cross, Salvation

Army, and National Weather Service

C. RACES supports the National Traffic System

D. RACES is a part of the National Emergency Warning System

~~

T8B04 (B)

What is the primary function of ARES in relation to emergency activities?

A. ARES organizations are restricted to serving local, state, and federal government emergency management agencies

B. ARES supports agencies like the Red Cross, Salvation

Army, and National Weather Service

C. ARES groups work only with local school districts

D. ARES supports local National Guard units

~~

T8B05 (C) [97.407(a)]

What organization must you register with before you can participate in RACES activities?

A. A local amateur radio club

B. A local racing organization

C. The responsible civil defense organization

D. The Federal Communications Commission

~~

T8B06 (B)

What is necessary before you can join an ARES group?

A. You are required to join the ARRL

B. You must have an amateur radio license

C. You must have an amateur radio license and have Red Cross CPR training

D. You must register with a civil defense organization

~~

T8B07 (D)

What could be used as an alternate source of power to operate radio equipment during emergencies?

A. The battery in a car or truck

B. A bicycle generator

C. A portable solar panel

D. All of these answers are correct

~~

T8B08 (B) [97.403, 97.405(a),(b)]

When can you use non-amateur frequencies or equipment to call for help in a situation involving immediate danger to life or property?

A. Never; your license only allows you to use the frequencies authorized to your class of license

B. In a genuine emergency you may use any means at your disposal to call for help on any frequency

C. When you have permission from the owner of the set

D. When you have permission from a police officer on the scene

~~

T8B09 (C)

Why should casual conversation between stations during a public service event be avoided?

A. Such chatter is often interesting to bystanders

B. Other listeners might overhear personal information

C. Idle chatter may interfere with important traffic

D. You might have to change batteries more often

~~

T8B10 (B)

What should you do if a reporter asks to use your amateur radio transceiver to make a news report?

A. Allow the use but give your call sign every 10 minutes

B. Advise them that the FCC prohibits such use

C. Tell them it is OK as long as you do not receive compensation

D. Tell the reporter that you must approve the material beforehand

~~

T8B11 (C) [97.403, 97.405(a),(b)]

When can you use a modified amateur radio transceiver to transmit on the local fire department frequency?

A. When you are helping the Fire Department raise money

B. Only when the Fire Department is short of regular equipment

C. In a genuine emergency you may use any means at your disposal to call for help on any frequency

D. When the local Fire Chief has given written permission

~~

T8C - Net operations, responsibilities of the net control station, message handling,  interfacing with public safety officials  - 1 exam question

T8C01 (A)

Which type of traffic has the highest priority?

A. Emergency traffic

B. Priority traffic

C. Health and welfare traffic

D. Routine traffic

~~

T8C02 (B)

What type of messages should not be transmitted over amateur radio frequencies during emergencies?

A. Requests for supplies

B. Personal information concerning victims

C. A schedule of relief operators

D. Estimates of how much longer the emergency will last

~~

T8C03 (C)

What should you do to minimize disruptions to an emergency traffic net once you have checked in?

A. Whenever the net frequency is quiet, announce your call sign and location

B. Move 5 kHz away from the net's frequency and use high power to ask other hams to keep clear of the net frequency

C. Do not transmit on the net frequency until asked to do so by the net control station

D. Wait until the net frequency is quiet, then ask for any

emergency traffic for your area

~~

T8C04 (B)

What is one thing that must be included when passing emergency messages?

A. The call signs of all the stations passing the message

B. The name of the person originating the message

C. A status report

D. The message title

~~

T8C05 (A)

What is one way to reduce the chances of casual listeners overhearing sensitive emergency traffic?

A. Pass messages using a non-voice mode such as packet radio or Morse code

B. Speak as rapidly as possible to reduce your on-air time

C. Spell out every word using phonetics

D. Restrict transmission of messages to the hours between midnight and 4:00 AM

~~

T8C06 (C)

What is of primary importance for a net control station?

A. A dual-band transceiver

B. A network card

C. A strong and clear signal

D. The ability to speak several languages

~~

T8C07 (B)

What should the net control station do if someone breaks in with emergency traffic?

A. Ask them to wait until the roll has been called

B. Stop all net activity until the emergency has been handled

C. Ask the station to call the local police and then resume

normal net activities

D. Ask them to move off your net frequency immediately

~~

T8C08 (C)

What should you do if a large scale emergency has just occurred and no net control station is available?

A. Wait until the assigned net control station comes on the air and pass your traffic when called

B. Transmit a call for help and hope someone will hear you

C. Open the emergency net immediately and ask for check-ins

D. Listen to the local NOAA weather broadcast to find out how long the emergency will last

~~

T8C09 (D)

What is the preamble of a message?

A. The first paragraph of the message text

B. The message number

C. The priority handling indicator for the message

D. The information needed to track the message as it passes

through the amateur radio traffic handling system

~~

T8C10 (A)

What is meant by the term "check" in reference to a message?

A. The check is a count of the number of words in the message

B. The check is the value of a money order attached to the message

C. The check is a list of stations that have relayed the message

D. The check is a box on the message form that tells you the message was received

~~

T8C11 (B)

What is the recommended guideline for the maximum number of words to be included in the text of an emergency message?

A. 10 words

B. 25 words

C. 50 words

D. 75 words

~~

SUBELEMENT T0 – Electrical and RF Safety – 3 exam questions – 3 groups

T0A – AC power circuits, hazardous voltages, fuses and  circuit breakers, grounding, lightning protection, battery safety, electrical code compliance – 1 exam question

T0A09 (C)

What is one way to recharge a 12-volt battery if the commercial power is out?

A. You cannot recharge a battery unless the power is back on

B. Add water to the battery

C. Connect the battery to a car's battery and run the engine

D. Take your battery to the utility company for a recharge

~~